--------------------------------------------------------------------------------

--

--   AM2901 Benchmark

--

-- Source: AM2901 Data Book

--

-- author: Indraneel Ghosh

--         University Of California, Irvine, CA 92717

--

-- Written on Jan 1, 1992

--

--------------------------------------------------------------------------------

TESTING THE Am2901 MODELS.

PORTS

I	input

D	input

Aadd	input

Badd	input

C0	input

OEbar	input

RAM0	input

RAM3	input

Q0	input

Q3	input

Y	output

C4	output

Gbar	output

Pbar	output

OVR	output

F3	output

F30	output

RAM0	output

RAM3	output

Q0	output

Q3	output

Details about these ports are mentioned in the "am2901.doc" file.

OVERALL TESTING STRATEGY

In testing the Am2901 models, the overall strategy adopted is to

test each "hardware" component (e.g. alu, RAM etc.)  using

sequences of test vectors.

The components are tested in different modes with test vectors designed

to detect "stuck-at-0" and "stuck-at-1" faults at various points in the

hardware.

Paths are also tested for "stuck-at-0" and "stuck-at-1" faults at any

point on the path. Paths start at some register (or external data input),

go through the ALU and end at a register.

Because the components are not being tested in isolation, we need to

set up input values at input ports of the chip and propagate them to

the input of that component. Also, the output of a component has to

be propagated to the output ports of the chip.

So, an error indication generated while running the test vectors for a

particular component might result from faults in other components that

are in the propagation paths.

However, the overall error indication pattern, generated by the simulation,

will provide a better pointer to the location of the fault. It will exclude

some possible fault locations, in most cases.

TESTING THE COMPONENTS

Now, we discuss the testing of the individual components.

************************

*                      *

* TEST VECTORS FOR ALU *

*                      *

************************

Each of the eight functions of the 4-bit alu is individually tested.

The function to be selected depends on the value of ( I5 I4 I3 ).

In all cases, each bit of the 4-bit functional unit is tested for

"stuck-at-0" and "stuck-at-1" faults.

ADDITION  R + S : Each of the 4 full-adders is tested for "stuck-at-0"

and "stuck-at-1" faults. So each full adder gets each

of the inputs (0,0) (0,1) (1,1) (1,0) at least once

and both the sum and carry values are checked, along

with the other ALU status signals OVR, F3, F30, P_bar,

G_bar. This sort of testing is done with both values

of carry-in C0.

To test the carry propagation in each of the 4 full

adders, 4 additional test vectors are used.

SUBTRACTION S - R : Each of the 4 full-adders is tested for "stuck-at-0"

and "stuck-at-1" faults. So each full adder gets each

of the inputs (0,0) (0,1) (1,1) (1,0) at least once

and both the sum and borrow values are checked, along

with the other ALU status signals OVR, F3, F30, P_bar,

G_bar. This sort of testing is done with both values

of borrow-in C0.

To test the borrow propagation in each of the 4 full

adders, 4 additional test vectors are used.

SUBTRACTION R - S : Each of the 4 full-adders is tested for "stuck-at-0"

and "stuck-at-1" faults. So each full adder gets each

of the inputs (0,0) (0,1) (1,1) (1,0) at least once

and both the sum and borrow values are checked, along

with the other ALU status signals OVR, F3, F30, P_bar,

G_bar. This sort of testing is done with both values

of borrow-in C0.

To test the borrow propagation in each of the 4 full

adders, 4 additional test vectors are used.

OR (R or S) : Each of the 4 OR gates is tested for "stuck-at-0"

and "stuck-at-1" faults with inputs (0,0) (0,1) (1,1)

(1,0).

AND (R and S) : Each of the 4 AND gates is tested for "stuck-at-0"

and "stuck-at-1" faults with inputs (0,0) (0,1) (1,1)

(1,0).

AND (not(R) and S) : Each of the 4 AND gates is tested for "stuck-at-0"

and "stuck-at-1" faults with inputs (0,0) (0,1) (1,1)

(1,0).

XOR (R xor S) : Each of the 4 XOR gates is tested for "stuck-at-0"

and "stuck-at-1" faults with inputs (0,0) (0,1) (1,1)

(1,0).

XNOR (R xnor S) : Each of the 4 XNOR gates is tested for "stuck-at-0"

and "stuck-at-1" faults with inputs (0,0) (0,1) (1,1)

(1,0).

************************

*                      *

* TEST VECTORS FOR RAM *

*                      *

************************

The RAM has 16 memory locations which are 4-bits wide. It also has an

up/down shifter at its input. It has two ports A and B. The address

at port A is only for reading. The address at B port is for reading

and writing.

WRITING 0's AND 1's

IN RAM LOCATIONS : Each memory location has all 0's and all 1's

written in it using the B port address. The

location written to is then read using the

A port. It is read again using the B port.

This tests for "stuck-at" faults in all

memory locations and ports.

TESTING RAM

SHIFTERS : The up shift and down shift operations are

tested by shifting a '0' and then a '1' in

each case. In one test vector a '0' is shifted

in and a '1' is shifted out. In another test

vector, its vice versa.

Since, two values of ( I8 I7 I6 ) cause

an up-shift operation and two cause a down

shift operation, all these test vectors are

executed once for each value.

*******************************

*                             *

* TEST VECTORS FOR Q-REGISTER *

*                             *

*******************************

The Q register is 4-bits wide and has an up/down shifter at its

input.

WRITING 0's AND 1's : All 0's and all 1's are written to the

Q-register and then read from it to

check for "stuck-at" faults.

TESTING Q-REGISTER

SHIFTERS : The up shift and down shift operations are

tested by shifting a '0' and then a '1' in

each case. In one test vector a '0' is shifted

in and a '1' is shifted out. In another test

vector, its vice versa.

Since, two values of ( I8 I7 I6 ) cause

an up-shift operation and two cause a down

shift operation, all these test vectors are

executed once for each value.

****************************************

*                                      *

* TEST VECTORS FOR ALU SOURCE-SELECTOR *

*                                      *

****************************************

The ALU source-selector has 8 modes of operation depending on the

value of ( I2 I1 I0 ). In each mode, sources are selected for the

2 ALU inputs.

TESTING FOR ALU SOURCE-SELECT ( R = A, S = Q) :

Test vectors use all 0's and all 1's for the selected sources of

ALU inputs to check for "stuck-at" faults.

TESTING FOR ALU SOURCE-SELECT ( R = A, S = B) :

Test vectors use all 0's and all 1's for the selected sources of

ALU inputs to check for "stuck-at" faults.

TESTING FOR ALU SOURCE-SELECT ( R = 0, S = Q) :

Test vectors use all 0's and all 1's for the selected sources of

ALU inputs to check for "stuck-at" faults.

TESTING FOR ALU SOURCE-SELECT ( R = 0, S = B) :

Test vectors use all 0's and all 1's for the selected sources of

ALU inputs to check for "stuck-at" faults.

TESTING FOR ALU SOURCE-SELECT ( R = 0, S = A) :

Test vectors use all 0's and all 1's for the selected sources of

ALU inputs to check for "stuck-at" faults.

TESTING FOR ALU SOURCE-SELECT ( R = D, S = A) :

Test vectors use all 0's and all 1's for the selected sources of

ALU inputs to check for "stuck-at" faults.

TESTING FOR ALU SOURCE-SELECT ( R = D, S = Q) :

Test vectors use all 0's and all 1's for the selected sources of

ALU inputs to check for "stuck-at" faults.

TESTING FOR ALU SOURCE-SELECT ( R = D, S = 0) :

Test vectors use all 0's and all 1's for the selected sources of

ALU inputs to check for "stuck-at" faults.

************************************

*                                  *

* TEST VECTORS FOR OUTPUT-SELECTOR *

*                                  *

************************************

The ALU output selector selects the output according to the value

of ( I8 I7 I6 ). So, it has 8 modes of operation.

TESTING OUTPUT FOR (I8 I7 I6 = 000) : Test for "stuck-at" faults

using all 0's and all 1's.

Also, test for the case when

output is tristated.

TESTING OUTPUT FOR (I8 I7 I6 = 001) : Test for "stuck-at" faults

using all 0's and all 1's.

Also, test for the case when

output is tristated.

TESTING OUTPUT FOR (I8 I7 I6 = 010) : Test for "stuck-at" faults

using all 0's and all 1's.

Also, test for the case when

output is tristated.

TESTING OUTPUT FOR (I8 I7 I6 = 011) : Test for "stuck-at" faults

using all 0's and all 1's.

Also, test for the case when

output is tristated.

TESTING OUTPUT FOR (I8 I7 I6 = 100) : Test for "stuck-at" faults

using all 0's and all 1's.

Also, test for the case when

output is tristated.

TESTING OUTPUT FOR (I8 I7 I6 = 101) : Test for "stuck-at" faults

using all 0's and all 1's.

Also, test for the case when

output is tristated.

TESTING OUTPUT FOR (I8 I7 I6 = 110) : Test for "stuck-at" faults

using all 0's and all 1's.

Also, test for the case when

output is tristated.

TESTING OUTPUT FOR (I8 I7 I6 = 111) : Test for "stuck-at" faults

using all 0's and all 1's.

Also, test for the case when

output is tristated.

**************************

*                        *

* TEST VECTORS FOR PATHS *

*                        *

**************************

Paths start at some register (or external data input), go through the ALU

and end at a register.

We can classify paths into 2 types :

-- Those that end at the Q-register

-- Those that end at the RAM

*** TESTING PATHS ( "ALU-SOURCES" --> F --> Q ) ***

These are the paths that end at the Q-register. There are 8 types of

paths depending on the ALU sources they start at :-

PATH TEST WITH ALU-SOURCE ( R = A , S = Q ) : The path is tested for

"stuck-at" faults at any

point using all 1's and

all 0's.

PATH TEST WITH ALU-SOURCE ( R = A , S = B ) : The path is tested for

"stuck-at" faults at any

point using all 1's and

all 0's.

PATH TEST WITH ALU-SOURCE ( R = 0 , S = Q ) : The path is tested for

"stuck-at" faults at any

point using all 1's and

all 0's.

PATH TEST WITH ALU-SOURCE ( R = 0 , S = B ) : The path is tested for

"stuck-at" faults at any

point using all 1's and

all 0's.

PATH TEST WITH ALU-SOURCE ( R = 0 , S = A ) : The path is tested for

"stuck-at" faults at any

point using all 1's and

all 0's.

PATH TEST WITH ALU-SOURCE ( R = D , S = A ) : The path is tested for

"stuck-at" faults at any

point using all 1's and

all 0's.

PATH TEST WITH ALU-SOURCE ( R = D , S = Q ) : The path is tested for

"stuck-at" faults at any

point using all 1's and

all 0's.

PATH TEST WITH ALU-SOURCE ( R = D , S = 0 ) : The path is tested for

"stuck-at" faults at any

point using all 1's and

all 0's.

PATH TEST WITH ALU-SOURCE ( R = D , S = 0 ) : The path is tested for

"stuck-at" faults at any

point using all 1's and

all 0's.

*** TESTING PATHS ( "ALU-SOURCES" --> F --> B ) ***

These are the paths that end at the RAM. There are 8 types of

paths depending on the ALU sources they start at :-

PATH TEST WITH ALU-SOURCE ( R = A , S = Q ) : The path is tested for

"stuck-at" faults at any

point using all 1's and

all 0's.

PATH TEST WITH ALU-SOURCE ( R = A , S = B ) : The path is tested for

"stuck-at" faults at any

point using all 1's and

all 0's.

PATH TEST WITH ALU-SOURCE ( R = 0 , S = Q ) : The path is tested for

"stuck-at" faults at any

point using all 1's and

all 0's.

PATH TEST WITH ALU-SOURCE ( R = 0 , S = B ) : The path is tested for

"stuck-at" faults at any

point using all 1's and

all 0's.

PATH TEST WITH ALU-SOURCE ( R = 0 , S = A ) : The path is tested for

"stuck-at" faults at any

point using all 1's and

all 0's.

PATH TEST WITH ALU-SOURCE ( R = D , S = A ) : The path is tested for

"stuck-at" faults at any

point using all 1's and

all 0's.

PATH TEST WITH ALU-SOURCE ( R = D , S = Q ) : The path is tested for

"stuck-at" faults at any

point using all 1's and

all 0's.

PATH TEST WITH ALU-SOURCE ( R = D , S = 0 ) : The path is tested for

"stuck-at" faults at any

point using all 1's and

all 0's.

<div align="center"><br /><script type="text/javascript"><!--

google_ad_client = "pub-7293844627074885";

//468x60, Created at 07. 11. 25

google_ad_slot = "8619794253";

google_ad_width = 468;

google_ad_height = 60;

//--></script>

<script type="text/javascript" src="http://pagead2.googlesyndication.com/pagead/show_ads.js">

</script><br />&nbsp;</div>