--------------------------------------------------------------------------------

--

--   Intel 8251 Benchmark

--

-- Source:  Intel Data Book

--

-- VHDL Benchmark author Indraneel Ghosh

--                       University Of California, Irvine, CA 92717

--

-- Developed on April 7, 92

--

--------------------------------------------------------------------------------

OVERALL TESTING STRATEGY

In testing the functionality of the 8251, we mainly concentrate on testing

its main operational modes :

(1) Synchronous transmission

(2) Asynchronous transmission

(3) Synchronous receive (External Synchronization)

(4) Synchronous receive (Internal Synchronization)

(5) Asynchronous receive

Functional details peculiar to each mode are tested in each case, among

others.

We try to test various ports and signals in both their complimentary forms

(1 and 0) which is analogous to testing for "stuck-at" faults in synthesised

hardware.

*************************************

*				    *

* (1) Synchronous transmission mode *

*		  		    *

*************************************

The strategy is to transmit complementary values of all data bits and parity

bit at least once. Parity is tested for even and odd mode. Also test for

no parity.

To test Single-SYNC and Double-SYNC modes, we let the transmission buffer

remain empty at some times so that SYNC character(s) are transmitted.

In Single-SYNC mode, the transmitter is made to send the same SYNC1

character twice consecutively, to check Single SYNC operation.

Both SYNC1 and SYNC2 are tested for complementary values of all data bits.

These tests are performed for all character lengths ( 5, 6, 7, 8 bits).

**************************************

*	 			     *

* (2) Asynchronous transmission mode *

*		  		     *

**************************************

The strategy is to transmit complementary values of all data bits and parity

bit at least once. Parity is tested for even and odd mode. Also test for

no parity.

Check for start bit, appropriate number of stop bits at all baud rates.

Test for all possible baud rates (1X, 16X, 64X)

Test for all possible numbers of stop bits (1, 1.5, 2)

These tests are performed for all character lengths ( 5, 6, 7, 8 bits).

*******************************************************

*	 			     		      *

* (3) Synchronous receive (External Synchronization)  *

*		  		     		      *

*******************************************************

First, we check the external synchronization scheme by making the

receiver enter hunt mode and then sending a rising edge on SYNDET_BD.

The strategy is to receive complementary values of all data bits and parity

bit at least once. Parity is tested for even and odd mode. Also test for

no parity.

Also, we test the detection of parity error (for both values of parity in

even and odd modes) by sending erroneous parity bits. Also, check for

overrun error by not reading the received data in time.

These tests are performed for all character lengths ( 5, 6, 7, 8 bits).

*******************************************************

*	 			     		      *

* (4) Synchronous receive (Internal Synchronization)  *

*		  		     		      *

*******************************************************

First, we check the internal synchronization scheme by making the

receiver enter hunt mode and then sending sync characters. These tests

are performed for both Single-Sync and Double-Sync and for complementary

values of SYNC1 and SYNC2.

The strategy is to receive complementary values of all data bits and parity

bit at least once. Parity is tested for even and odd mode. Also test for

no parity.

Also, we test the detection of parity error (for both values of parity in

even and odd modes) by sending erroneous parity bits. Also, check for

overrun error by not reading the received data in time.

These tests are performed for all character lengths ( 5, 6, 7, 8 bits).

*******************************************************

*	 			     		      *

* (5) Asynchronous receive 			      *

*		  		     		      *

*******************************************************

In the Asynchronous mode, we test the False Start-Bit Detection Scheme

(for baud rate 16X and 64X) by sending false start bits ( send TxD low

but make it go high before half the baud cycle).

The strategy is to receive complementary values of all data bits and parity

bit at least once. Parity is tested for even and odd mode. Also test for

no parity.

Also, we test the detection of parity error (for both values of parity in

even and odd modes) by sending erroneous parity bits. Also, check for

overrun error by not reading the received data in time.

Also check for framing error detection by sending a low signal on RxD when

stop bit (high) is expected.

Test for all possible baud rates (1X, 16X, 64X)

Test for all possible numbers of stop bits (1, 1.5, 2)

These tests are performed for all character lengths ( 5, 6, 7, 8 bits).

Testing Strategy :

----------------

Each test vector line is equivalent to one clock cycle.

Since the 8251 has three clocks,

(

internal device clock -- "clk"

transmitter clock     -- "TxC_BAR"

receiver clock 	      -- "RxC_BAR"

)

it was convenient to put the test vectors in three CONCURRENTLY EXECUTING

processes ("main_sim", "Tx_sim", "Rx_sim") each corresponding to one of the

respective clocks.

(

process "main_sim" -- "clk"

process "Tx_sim"   -- "TxC_BAR"

process "Rx_sim"   -- "RxC_BAR"

)

Each test vector line is translated and included into ONE of these

concurrent processes. To signify which concurrent process the the line

should be included in, we include one of three characters ("M", "T", "R")

at the beginning of each test vector line.

(

"M" -- include it in process "main_sim"

"T" -- include it in process "Tx_sim"

"R" -- include it in process "Rx_sim"

)

The 8251 data-sheets specify that "clk" must be at least 4 times faster

than either "RxC_BAR" or "TxC_BAR". I have assumed that "clk" is 4 times

faster than each of them. So, each "TxC_BAR" or "RxC_BAR" cycle is

equivalent to 4 "clk" cycles. Therefore, to ensure that time advances

equally in all 3 concurrent processes ("main_sim", "Tx_sim" and "Rx_sim"),

we always have, in each "OVERALL-CYCLE" :

(a) 1 line with "T" ( equivalent to 1 "TxC_BAR" cycle)

(b) 1 line with "R" ( equivalent to 1 "RxC_BAR" cycle)

(c) 4 lines with "M" ( equivalent to 4 "clk" cycles)

It is not neccessary to activate any inputs or check any outputs in every

test vector line.

Also, if a different clock ratio ("clk" to "RxC_BAR" / "TxC_BAR") is

desired, that can be achieved by simply changing the ratio of "T", "R"

and "M" lines in each "OVERALL-CYCLE".

Values corresponding to ports are written in the exact order in which

port names are mentioned in the Port Name Line ( Type [a], Line {I} ).

In case we dont want to mention any value for a particular port in that

line, we put in one or more dashes ( - ) in that position corresponding

to that port. Values are separated by the colon as a port-separator.

Values in the input-port positions in Type [b] lines are assigned to the

corresponding input port. Only the "clocking" is implicit in the

translation process for each test vector and the clocks ("clk",

"RxC_BAR" and "TxC_BAR" ) are not specifically mentioned among the input

ports.

Values in the output-port positions in Type [b] lines are compared to the

actual output values appearing at the corresponding output ports when we

run the simulation. If these two values dont match, then an "Assert"

statement is generated to show that an error has been detected.

--------------------------------------------------------------------------

<div align="center"><br /><script type="text/javascript"><!--

google_ad_client = "pub-7293844627074885";

//468x60, Created at 07. 11. 25

google_ad_slot = "8619794253";

google_ad_width = 468;

google_ad_height = 60;

//--></script>

<script type="text/javascript" src="http://pagead2.googlesyndication.com/pagead/show_ads.js">

</script><br />&nbsp;</div>