----------------------------------------------------------------------------------

--   Kalman Filter Benchmark

--

-- Source:  The model, test patterns and part of the documentation

--          were obtained from Cleland O Newton at DRA Malvern, UK.

--

-- author: Champaka Ramachandran

--         University Of California, Irvine, CA 92717

--         champaka@balboa.eng.uci.edu

--

-- Written on Aug 24th, 92

--

----------------------------------------------------------------------------------

Testing Strategy :

------------------

At the start of the testing process, the matrices A, G and K are loaded.

All the input data is shown in integer, as it is easy to comprehend.

Since, the model needs a bit vector, conversion is done at the outset.

Part 1

------

Initialization is done so that, no data is written onto any unwanted

adresses.

Addr <= 0;

Cexec <= '0';

Vector_type <= "000";

Part 2

------

We have made use of the fact that A is upper diagonal and sparse. So, we

need to compute only the non-zero values.

The vector type indicates that Matrix A is being loaded. Address indicates

the matrix entry.

Vector_type <= "001";

i := 0;

while (i < 16) loop

j := 0;

while (j < 16) loop

if (i = j) then

Input_Vector <= SIGNED_INT_TO_BIN(-900, 16);

else

if (j - 9 = i) then

Input_Vector <= SIGNED_INT_TO_BIN(10, 16);

else

Input_Vector <= SIGNED_INT_TO_BIN(0, 16);

end if;

end if;

Addr <= i * 16 + j;

wait for 1 ns;

j := j + 1;

end loop;

i := i + 1;

end loop;

Part 3

------

In the case of matrix K, Almost all values are same except the diagonal element

and some zero occurances. The vector type indicates that matrix K is being set.

Vector_type <= "010";

i := 0;

while (i < 16) loop

j := 0;

while (j < 16) loop

if (i = j) and (j < 13) then

Input_Vector <= SIGNED_INT_TO_BIN(75, 16);

else

if (i >= 13) and (i - 13 = j) then

Input_Vector <= SIGNED_INT_TO_BIN(75, 16);

else

if (j < 13) then

Input_Vector <= SIGNED_INT_TO_BIN(67, 16);

else

Input_Vector <= SIGNED_INT_TO_BIN(0, 16);

end if;

end if;

end if;

Addr <= i * 16 + j;

wait for 1 ns;

j := j + 1;

end loop;

i := i + 1;

end loop;

Part 4

------

In the case of matrix G, most of the entries are same except some anomalies.

Vector_type <= "011";

i := 0;

while (i < 4) loop

j := 0;

while (j < 16) loop

Input_Vector <= SIGNED_INT_TO_BIN(358, 16);

if (i = 0) and (j = 0) then

Input_Vector <= SIGNED_INT_TO_BIN(328, 16);

end if;

if (i = 1) and (j = 5) then

Input_Vector <= SIGNED_INT_TO_BIN(328, 16);

end if;

if (i = 2) and (j = 9) then

Input_Vector <= SIGNED_INT_TO_BIN(328, 16);

end if;

if (i = 3) and (j = 12) then

Input_Vector <= SIGNED_INT_TO_BIN(328, 16);

end if;

Addr <= i * 16 + j;

wait for 1 ns;

j := j + 1;

end loop;

i := i + 1;

end loop;

Part 5

------

There exists 2 types of test sequences. Four different run of test vectors was

followed by a run of four identical inputs. This is done to demonstrate the

convergence of the state vector estimation process. The output values are seen

to match the input values at indices 1 to 4, for the given set of values on the

input matrices.  An example for setting input vector Y is shown

--

-- Pattern #1  Setting Input Y (1)

--

Vector_type <= "100";

Input_Vector <= SIGNED_INT_TO_BIN(501, 16);

Addr <= 0;

wait for 1 ns;

Input_Vector <= SIGNED_INT_TO_BIN(550, 16);

Addr <= 1;

wait for 1 ns;

Input_Vector <= SIGNED_INT_TO_BIN(504, 16);

Addr <= 2;

wait for 1 ns;

Input_Vector <= SIGNED_INT_TO_BIN(-500, 16);

Addr <= 3;

wait for 1 ns;

Input_Vector <= SIGNED_INT_TO_BIN(451, 16);

Addr <= 4;

wait for 1 ns;

Input_Vector <= SIGNED_INT_TO_BIN(650, 16);

Addr <= 5;

wait for 1 ns;

Input_Vector <= SIGNED_INT_TO_BIN(515, 16);

Addr <= 6;

wait for 1 ns;

Input_Vector <= SIGNED_INT_TO_BIN(-481, 16);

Addr <= 7;

wait for 1 ns;

Input_Vector <= SIGNED_INT_TO_BIN(-541, 16);

Addr <= 8;

wait for 1 ns;

Input_Vector <= SIGNED_INT_TO_BIN(410, 16);

Addr <= 9;

wait for 1 ns;

Input_Vector <= SIGNED_INT_TO_BIN(520, 16);

Addr <= 10;

wait for 1 ns;

Input_Vector <= SIGNED_INT_TO_BIN(421, 16);

Addr <= 11;

wait for 1 ns;

Input_Vector <= SIGNED_INT_TO_BIN(-491, 16);

Addr <= 12;

wait for 1 ns;

Cexec <= '1';

wait for 1 ns;

Part 6

------

This part consists of sanity check vectors. We test for all inputs

being positive, all being negative, and some being positive and negative.

We then vary the size of the inputs starting from order 1 to order 10, order 100

and order 1000.

The next test is to intermingle different order inputs among the different

inputs of Y. This test is carried out on all postive, all negative and some

positive- negative inputs.

The final test alternates postive and negative inputs among order 10, 100 and 1000

<div align="center"><br /><script type="text/javascript"><!--

google_ad_client = "pub-7293844627074885";

//468x60, Created at 07. 11. 25

google_ad_slot = "8619794253";

google_ad_width = 468;

google_ad_height = 60;

//--></script>

<script type="text/javascript" src="http://pagead2.googlesyndication.com/pagead/show_ads.js">

</script><br />&nbsp;</div>