----------------------------------------------------------------------------------

-- Radix-512 Divider Benchmark

--

-- Source:  "Division and Square Root: Digit-Recurrence Algorithms and

--           Implementations" M.D. Ergegovac, T. Lang

--

-- VHDL Benchmark author: Alberto Nannarelli on Jan 28 1994

--

-- Documentation

--

--------------------------------------------------------------------------------

The division to be performed is a division between double precision

floating point numbers, namely x and d, represented in IEEE standard

notation (mantissa 53 bits, sign 1 bit, exponent 10 bits).

In this algorithm only the mantissa is calculated being sign and exponent

of the quotient attainable just by comparing and shifting.

Thus d must be in the range:

1/2 =< d < 1

and x:

1/4 =< x < d

the quotient q

x

--- = q

d

in the range:

1/4 =< x < d

The Digit-Recurrence Algorithms uses a radix r = 512 = 2^9

and 9 bits of the quotient are produced every iteration.

Because of convergence problems, the divider must be within a determined

range. To achieve this, We scale both operands by a quantity M so that:

z = M*d

and

X = M*x

and the condition to be satisfied is:

1 - alfa < z < 1 + beta

where both alfa and beta are functions of the radix r.

The recurrence to be executed is:

w[j+1] = r*w[j] - q(j+1)*z		j = 1, 2 ..... (n.bit)/log2(r)

q(j) = floor{ r*w[j] + 1/2 }

where

w[i] is the residual after iteration i, w[0]=X;

q(i) are the 9 quotient bits generated at iteration i;

The addition are made by a carry save adder (CSA), a circuit that generates

2 outputs (sum and carry) and speeds up the iterations (no carry propagation)

in the hardware implementation.

At the end if the final residual is positive, the quotient must be rounded

by adding 1 to its mantissa.

<div align="center"><br /><script type="text/javascript"><!--

google_ad_client = "pub-7293844627074885";

//468x60, Created at 07. 11. 25

google_ad_slot = "8619794253";

google_ad_width = 468;

google_ad_height = 60;

//--></script>

<script type="text/javascript" src="http://pagead2.googlesyndication.com/pagead/show_ads.js">

</script><br />&nbsp;</div>