--------------------------------------------------------------------------------

--

--   Prawn CPU Benchmark Documentation

--

-- Derived from

--           Parwan : a reduced processor

--           from Chapter 9 of NAVABI : "VHDL: Analysis and Modeling of

--           Digital Systems" McGraw-Hill,Inc. 1993

--

-- Additional Features: interrupt, stack, and conditions for

--           conditional branch have been added.

--

-- Author: Tadatoshi Ishii

--         Information and Computer Science,

--         University Of California, Irvine, CA 92717

--

-- Written on Nov 1, 1992

--

--------------------------------------------------------------------------------

================================================================================

(1) Introduction

Prawn is a simple eight-bit microprocessor based on the sample processor

described in Chapter 9 of "VHDL : Analysis and Modeling of Digital Systems"

by Z. Navabi, McGraw-Hill,Inc. 1993.  We have added some features such as

interrupt, stack and some conditions for conditional branch to the example in

the book.

Some of the requirements for this benchmark are listed below:

1) All basic CPU functions should be represented.

2) It should use a simple architecture which can be easily mapped into

hardware with existing synthesis tools.

3) The design must be simple enough to allow manual verification.

4) The design should have a complete set of test vectors that exercises all

processor functions.

================================================================================

(2) Prawn CPU

Based on the requirements, Prawn is an eight-bit processor which has an

8-bit Data Bus and a 12-bit Address Bus for external accesses.  It has a

limited number of arithmetic and logic instructions, several jump and branch

instructions, subroutine call instructions, and interrupt instructions.  Some

instructions have an addressing mode, which provides for direct and indirect

addressing.  Prawn has an accumulator, a reduced ALU, a shifter, program

counter, and five flags (interrupt enable, overflow, carry, zero, sign).  It

uses a designated address of memory as a stack pointer register.

(2.1) Ports

Table 1 : INPUT AND OUTPUT PORTS:

+----------+-------+-----+-----------------------------------------------------+

|    PORT  | TYPE  |BIT  |                                                     |

|          |       |WIDTH|      DESCRIPTION                                    |

+----------+-------+-----+-----------------------------------------------------+

|      clk |    in |  1  | clk                                                 |

+----------+-------+-----+-----------------------------------------------------+

|    reset |    in |  1  | reset (active HIGH)                                 |

+----------+-------+-----+-----------------------------------------------------+

| read-mem |   out |  1  | Read memory (active HIGH)                           |

+----------+-------+-----+-----------------------------------------------------+

|write-mem |   out |  1  | Write memory (active HIGH)                          |

+----------+-------+-----+-----------------------------------------------------+

|  databus | inout |  8  | Data input/output to/from chip                      |

+----------+-------+-----+-----------------------------------------------------+

|    adbus |   out | 12  | Address output from chip                            |

+----------+-------+-----+-----------------------------------------------------+

|interrupt |    in |  1  | interrupt input to chip (active HIGH)               |

+----------+-------+-----+-----------------------------------------------------+

|     inta |   out |  1  | interrupt acknowledge output from chip (active HIGH)|

+----------+-------+-----+-----------------------------------------------------+

(2.2) Block Diagram

Since we describe Prawn with a behavioral model, its implementation

could be realized with any kind of bus structure.  We present a sample Block

Diagram for the Prawn CPU to facilitate understanding of its behavior.

Figure 1 : Block Diagram of Prawn CPU

+----------------------------------------+

databus|  Memory                                |adbus

+<-->|   4096          F:00-FE Stack Area     |<---+

|    |   bytes         F:FF Stack Pointer Reg |    |

Memory            |    +----------------------------------------+    |

------------------------------------------------------------------------

Prawn CPU         |                                                  |

|                                                  |

-----+------+-----------------------------+                    |

^    |           ^                        |                    |

|    |           |                        |                    |

|    |       +---+------+                 |                    |

|    |       |   |      |                 |                    |

|    |       V   |      V                 |                    |

|    |     +---+ |  +---------------+     |                    |

|    |     |Acc| |  |Instruction Reg|     |                    |

|    |     +---+ |  +---------------+     |                    |

|    |       |   |      |                 |                    |

|    V       V   |      +--------------+  |                    |

|   ----   ----  |      |              |  |              +-----+

+-->\   \_/   /  |      V              |  |              |     |

|    \  ALU  /   |  +---------------+  |  |              V     |

|    |\_____/    |  |   SEQUENCER   |  |  |  +---------------+ |

|    |   |       |  |               |  |  |  |Program Counter| |

|    V   V       |  |               |  |  |  +---------------+ |

|  +-------+     |  +---------------+  |  +----------+   |     |

|  |SHIFTER|     |    ^ ^ ^ |   | |    +---------+   |   |     |

|  +-------+     |    | | | |   | |              V   V   V     |

|    |   |       |    | | | |   | |           +-------------+  |

|    |   +-->----+    | | | |   | |           | address reg |  |

|    V   V            | | | |   | |           +-------------+  |

|  +------+           | | | |   | |                  |         |

|  | FLAG |           | | | |   | V                  |         |

|  +------+           | | | |   V  read_mem          +---------+

|     |               | | | V   write_mem

+--<--+---->----------+ | | inta

| interrupt

reset

(2.3) Memory Organization

Prawn has a 12-bit address bus, which is partitioned into sixteen pages

of 256 bytes each.  The four most significant bits of the address are for the

page address and the remaining eight bits of the address are for the offset.

The addresses are usually written in hexadecimal.  The page address and

offset are separated by ":".  These page areas are not continuous.  The last

Page is reserved for the stack area.  The address F:FF is used for the stack

pointer.  Prawn's I/O is memory mapped.

(2.4) Instruction Set

Table 2 summarizes Prawn's instruction set.  The column labeled "Flags"

indicates flags used or set by individual instructions.  The column "Clock"

defines the number of clock cycles for each instruction.

Prawn has single byte instructions and double byte instructions.  Many

of the double byte instructions have two addressing modes: direct and

indirect.  For a double byte instruction, the 3 MSB of the first byte

determines its opcode, while the next bit determines direct(0) or indirect(1)

addressing.

The "RESERVED" instruction indicates the reserved instruction for future

extensions of Prawn.

This VHDL behavior model of Prawn treats undefined instructions as

No-Ops.  You may treat these undefined instructions as "don't care

instructions".  Prawn does not have a trap mechanism to detect these

"Undefined Instructions".

Table 2 : Instruction set

+------------------------------------------------------------------------------+

|Double Byte Instructions w/ direct/indirect address                           |

+--------+------------+------+---------------------+--------------+------------+

|Mnemonic|First Byte  |Second|Description	   |Flags(use/set)|Clock(d/ind)|

+--------+------------+------+---------------------+--------------+------------+

|LDA  loc|000 0/1 page|offset|ac <- memory	   |-----/---zs   |2/3         |

+--------+------------+------+---------------------+--------------+------------+

|AND  loc|001 0/1 page|offset|ac <- ac and memory  |-----/---zs   |2/3         |

+--------+------------+------+---------------------+--------------+------------+

|ADD  loc|010 0/1 page|offset|ac <- ac + memory	   |--c--/-vczs   |2/3         |

+--------+------------+------+---------------------+--------------+------------+

|SUB  loc|011 0/1 page|offset|ac <- ac - memory	   |--c--/-vczs   |2/3         |

+--------+------------+------+---------------------+--------------+------------+

|JMP  loc|100 0/1 page|offset|long range JMP, allow|-----/-----   |2/3         |

|        |            |      |advance of pc beyond the page limit |            |

+--------+------------+------+---------------------+--------------+------------+

|STA  loc|101 0/1 page|offset|memory <- ac	   |-----/-----   |2/3         |

+--------+------------+------+---------------------+--------------+------------+

+------------------------------------------------------------------------------+

|Double Byte Instructions w/ direct address				       |

+----------+----------+------+----------------------------+--------------+-----+

|Mnemonic  |First Byte|Second|Description		  |Flags(use/set)|Clock|

+----------+----------+------+----------------------------+--------------+-----+

|JSR    loc|110 0 page|offset|long range JMP to subroutine|-----/-----   |6    |

|          |          |      |allow advance of pc beyond the page limit  |     |

+----------+----------+-----------------------------------+--------------+-----+

|RESERVED  |110 1 0000|reserved for ESC code for future instructions	       |

+----------+----------+--------------------------------------------------------+

|UNDEF	   |110 1 0001|						               |

|UNDEF	   |110 1 0010|					      	               |

|UNDEF	   |110 1 0011|					     	               |

|UNDEF	   |110 1 0100|						               |

|UNDEF	   |110 1 0101|						               |

|UNDEF	   |110 1 0110|						               |

|UNDEF	   |110 1 0111|						               |

|UNDEF	   |110 1 1000|						               |

|UNDEF	   |110 1 1001|						               |

|UNDEF	   |110 1 1010|						               |

|UNDEF	   |110 1 1011|						               |

|UNDEF	   |110 1 1100|						               |

|UNDEF	   |110 1 1101|						               |

|UNDEF	   |110 1 1110|						               |

+----------+----------+--------------------------------------------------------+

+------------------------------------------------------------------------------+

|Single Byte Instruction						       |

+--------+----------+---------------------------+---------------+--------------+

|Mnemonic|First Byte|Description	        |Flags(use/set) |Clock         |

+--------+----------+---------------------------+---------------+--------------+

|JRT	 |110 1 1111|long range RET(from JSR)   |-----/-----    |5             |

+--------+----------+---------------------------+---------------+--------------+

|NOP	 |111 0 0000|No operation	        |-----/-----    |1             |

+--------+----------+---------------------------+---------------+--------------+

|CLA	 |111 0 0001|clear ac		        |-----/-----    |1             |

+--------+----------+---------------------------+---------------+--------------+

|CMA	 |111 0 0010|complement ac	        |-----/---zs    |1             |

+--------+----------+---------------------------+---------------+--------------+

|BRT     |111 0 0011|short range RET(from BSR)  |-----/-----    |4             |

+--------+----------+---------------------------+---------------+--------------+

|CMC	 |111 0 0100|complement carry flag      |--c--/--c--    |1             |

+--------+----------+---------------------------+---------------+--------------+

|STI	 |111 0 0101|set interrupt enable flag  |-----/i----    |1             |

+--------+----------+---------------------------+---------------+--------------+

|IRT	 |111 0 0110|RET from interrupt         |-----/ivczs    |6             |

+--------+----------+---------------------------+---------------+--------------+

|CLI	 |111 0 0111|clear interrupt enable flag|-----/i----    |1             |

+--------+----------+---------------------------+---------------+--------------+

|ASL	 |111 0 1000|arithmetic shift left	|-----/-vczs    |1             |

+--------+----------+---------------------------+---------------+--------------+

|ASR	 |111 0 1001|arithmetic shift right	|-----/---zs    |1             |

+--------+----------+---------------------------+---------------+--------------+

|ROL     |111 0 1010|rotate left	        |-----/---zs    |1             |

+--------+----------+---------------------------+---------------+--------------+

|ROR     |111 0 1011|rotate right               |-----/---zs    |1             |

+--------+----------+---------------------------+---------------+--------------+

|PUSH	 |111 0 1100|push ac to stack		|-----/-----    |4             |

+--------+----------+---------------------------+---------------+--------------+

|PUSHF	 |111 0 1101|push flag to stack		|ivczs/-----    |4             |

+--------+----------+---------------------------+---------------+--------------+

|POP	 |111 0 1110|pop ac from stack		|-----/-----    |4             |

+--------+----------+---------------------------+---------------+--------------+

|POPF	 |111 0 1111|pop flag from stack	|-----/ivczs    |4             |

+--------+----------+---------------------------+---------------+--------------+

+------------------------------------------------------------------------------+

|Branch Instruction w/ direct address   				       |

+----------+----------+------+-------------------------------+-----------+-----+

|Mnemonic  |First Byte|Second|Condition/Description          |Flags(u/s) |Clock|

+----------+----------+------+-------------------------------+-----------+-----+

|BRA_N	loc|111 1 0000|offset|if negative                    |----s/-----|2    |

+----------+----------+------+-------------------------------+-----------+-----+

|BRA_NN	loc|111 1 0001|offset|if not negative                |----s/-----|2    |

+----------+----------+------+-------------------------------+-----------+-----+

|BRA_Z	loc|111 1 0010|offset|if zero		             |---z-/-----|2    |

+----------+----------+------+-------------------------------+-----------+-----+

|BRA_NZ	loc|111 1 0011|offset|if not zero	             |---z-/-----|2    |

+----------+----------+------+-------------------------------+-----------+-----+

|BRA_C	loc|111 1 0100|offset|if carry		             |--c--/-----|2    |

+----------+----------+------+-------------------------------+-----------+-----+

|BRA_NC	loc|111 1 0101|offset|if not carry	             |--c--/-----|2    |

+----------+----------+------+-------------------------------+-----------+-----+

|BRA_HI	loc|111 1 0110|offset|if high		             |--cz-/-----|2    |

+----------+----------+------+-------------------------------+-----------+-----+

|BRA_LO	loc|111 1 0111|offset|if low		             |--cz-/-----|2    |

+----------+----------+------+-------------------------------+-----------+-----+

|BRA_V	loc|111 1 1000|offset|if overflow	             |-v---/-----|2    |

+----------+----------+------+-------------------------------+-----------+-----+

|BRA_NV	loc|111 1 1001|offset|if not overflow	             |-v---/-----|2    |

+----------+----------+------+-------------------------------+-----------+-----+

|BRA_LT	loc|111 1 1010|offset|if less than	             |-v--s/-----|2    |

+----------+----------+------+-------------------------------+-----------+-----+

|BRA_GE	loc|111 1 1011|offset|if greater than or equal       |-v--s/-----|2    |

+----------+----------+------+-------------------------------+-----------+-----+

|BSR    loc|111 1 1100|offset|short range JMP to subroutine  |-----/-----|5    |

|          |          |      |allow advance of pc within the page limit  |     |

+----------+----------+------+-------------------------------+-----------+-----+

|BRA	loc|111 1 1101|offset|always, short range JMP        |-----/-----|2    |

|          |          |      |allow advance of pc within the page limit  |     |

+----------+----------+------+-------------------------------+-----------+-----+

|BRA_LE	loc|111 1 1110|offset|if less than of equal          |-v-zs/-----|2    |

+----------+----------+------+-------------------------------+-----------+-----+

|BRA_GT	loc|111 1 1111|offset|if greater than                |-v-zs/-----|2    |

+----------+----------+------+-------------------------------+-----------+-----+

(2.5) Addressing Mode

If an instruction has a page operand and an offset operand for the

location of its data, it is in full addressing mode;  this mode can have both

direct and indirect addressing.  If an instruction has only an offset operand

for the location of its data, it is in page addressing mode, where the

effective address is obtained by concatenating the page address of the

instruction with the offset operand.  In the page addressing mode, an

instruction can only reference memory locations within the page where the

instruction appears.

In the direct addressing mode, the effective address is obtained by

concatenating the page operand with the offset operand of the instruction.

In the indirect addressing mode, the offset for the effective address is

fetched from the memory location of the address obtained by concatenating the

page operand and the offset operand of the instruction.  Because of the page

separation, the page part of the effective address is the same as the

instruction's page itself.

(2.6) Stack and Subroutine

The Prawn CPU treats the last page of memory (locations F:00 to F:FE) as

the stack area, and address F:FF as the stack pointer.  During initialization

of the Prawn CPU, the stack pointer must point to the memory address F:FE, so

that the subroutine, push, pop and interrupt mechanisms of the CPU can use

the stack automatically.  This initialization of the stack pointer is done by

loading 'FE' to the memory at location F:FF.

The JSR instruction includes the operation which pushes the return

address (both page and offset) to the stack.  The JRT instruction pops the

return address from stack and changes the control flow of the Prawn CPU to

its return point.  The BSR and BRT instructions push and pop only the offset

part of the return address.  BSR has only the offset part of address at which

the subroutine starts.  These two instruction are used for calling and

returning from subroutines in the same page as the instructions.

The PUSH and POP instructions also increment and decrement the stack

pointer, respectively.

The Prawn CPU has no trap mechanism to indicate a stack overflow or

underflow. The programmer should ensure that the stack does not overflow or

underflow.

(2.7) Interrupt

The Prawn CPU has 2 interrupt ports.  One is a "reset" and other is a

"interrupt".  Both interrupt ports are checked at the end of the current

instruction.  Of course, "reset" has higher priority than "interrupt".

When enbaled, the "reset" port stops all operations of the Prawn CPU.

It clears the PC, flags, and adbus.  It sets the output data on databus to

high impedance.  After "reset" is disabled, the Prawn CPU fetches its first

instruction from the memory location 0:00.

The "interrupt" port is activated by I/O systems, and stays high until

the Prawn CPU returns "inta", the interrupt acknowledge signal.  The Prawn

CPU activates "inta" after the operation of current instruction is finished

and if the interrupt enable flag is set.  The system that sends an

"interrupt" to the CPU should consider "inta" as "read_mem" for fetching the

start address of the interrupt service routine which services the interrupt

request.  Because the Prawn CPU has a 12-bit address, the CPU needs two

cycles to get the address.  The CPU sends "inta" twice with the same timing

as "read_mem".  The CPU should be given the page part of the start address

for the interrupt service routine at the first "inta", and the offset at the

second "inta".

After address is given to the CPU, Prawn pushes the return address and

flags onto its stack and clears the interrupt enable flag.  Then it changes

its flow of control to the interrupt service routine.

IRT should be used for returning from interrupt service routine.  It

pops the flags and return address from stack and changes the control flow of

Prawn to its original routine.

================================================================================

(3) VHDL behavioral model developed for the Prawn CPU

This behavioral model is written with MVL7 and one package which is also

written with MVL7. The following files are used:

"types.vhdl"		: definition & basic functions of MVL7

"MVL7_functions.vhdl"	: utility functions for MVL7

"cpu.vhdl"		: behavioral model of Prawn CPU as a single VHDL

process.

"lib.vhdl"		: package for behavioral model of Prawn CPU

"tst*.vhdl"             : behavioral memory models which contain programs

for verifying functionality of Prawn CPU

"sim.vhdl"              : connection model between memory model and Prawn CPU

================================================================================

(4) Testing strategy

The model of the Prawn CPU has been tested with the set of test vectors

contained in files having prefix "tst".

Each test vector file consists of two sets of memories and one process

for simulation control. The first set of memories, called the Working Set,

contains the program, data, and working areas for the Prawn CPU.  These are

executed and used by the Prawn CPU during simulation.  The second set, called

Expected Set, has the expected contents for the set of memories when a

correct simulation is finished.

The control process in test vector files controls the simulation by

itself.  After a simulation is started, the control process controls access

to the Working Set and continues Prawn's execution until the end of the test

vector is detected.  We employ the following method to detect the end of each

test vector.  When executed correctly, each simulation ends at an address

(say, ADDR1) containing an instruction that repeatedly jumps to itself.  If

an error is detected during simulation due to an illegal flag condition, the

simulation jumps to another address location (say, ADDR2) also containing an

instruction that repeatedly jumps to itself.  The control process stops

Prawn's execution by stopping the service of memory access when these special

addresses (ADDR1 and ADDR2) are accessed.

When Prawn is stopped, the control process compares the contents of the

two sets of memories, and terminates the simulation itself using a severity

function. If a difference is found between the two sets of memories, the

simulation is stopped with severity FAILURE.  If no difference is found,

which means no error, the simulation is stopped with severity ERROR.

The simulation time of each test vector can be calculated according to

the cycle time for execution of each Prawn CPU instruction, as described in

Table2. However, even if your synthesized model has different execution

cycles for any instruction, you need not change any test vectors.  The test

vectors are independent of changes in the interface timing between Prawn and

the memories, as well as changes in the cycle time of any instruction.  The

tests only check the contents of memories after execution is completed.

The Prawn CPU is simulated with one of these test vector files at each

time and both are connected by the VHDL model which is in the file named

"sim.vhdl". The VHDL model in "sim.vhdl" looks at the VHDL model of the test

vector file as "mem.vhdl".  After selecting a specific test vector file for

simulation, you must copy that file over to "mem.vhdl" and then start your

VHDL simulation.

The file labeled "Makefile" has a simulation script for the Zycad VHDL

simulator v1.0. This may help you learn how to simulate the Prawn CPU.

(4.1)  Test Sets

Each test vector file contains a program that specifically tries to

verify the functionality of some of Prawn's instructions.  Verification of

Prawn's overall functionality should at least exercise all of the test vector

files that are intended to cover all individual operations of the Prawn CPU.

================================================================================

(5) Tested Functions

The following test vector files have been simulated on the Prawn CPU

model using the Zycad VHDL simulator v1.0.

Test File 	Tested instructions

---------		-------------------

tstop1.vhdl	lda direct

sta direct

tstop2.vhdl	cla

cma

cmc

asl

asr

nop

tstop3.vhdl	and direct/indirect

tstop4.vhdl	add direct/indirect

tstop5.vhdl	sub direct/indirect

tstop6.vhdl	jmp direct/indirect intra-page/inter-page

tstop7.vhdl	ror

rol

tstop8.vhdl	push

pop

pushf

popf

tstop9.vhdl	bra_?? (14 types of conditions)

tstopA.vhdl	jsr

jrt

bsr

brt

sti

cli

pushf

popf

tstopB.vhdl	lda indirect

sta indirect

tstopC.vhdl	interrupt handling mechanism

tst1book.vhdl	program from NAVABI's book

tst2book.vhdl	program from NAVABI's book

================================================================================

<div align="center"><br /><script type="text/javascript"><!--

google_ad_client = "pub-7293844627074885";

//468x60, Created at 07. 11. 25

google_ad_slot = "8619794253";

google_ad_width = 468;

google_ad_height = 60;

//--></script>

<script type="text/javascript" src="http://pagead2.googlesyndication.com/pagead/show_ads.js">

</script><br />&nbsp;</div>