--------------------------------------------------------------------------------

--

-- RTPC CPU Benchmark :

--	Documentation of the RTPC-Model

--

-- Derived from

--	RTPC description written in verilog by Edward Czeck et al.

--	Most likely based on the IBM RT-PC Hardware Technical

--	Reference (c) 1985 (for RT PC model 10, 20, and 25)

--

-- Authors:

--	Alfred B. Thordarson (abth@ece.uci.edu)

--	and

--	Nikil Dutt, professor of CS and ECE

--	University Of California, Irvine, CA 92717

--

-- Changes:

--	Dec 1, 1993: File created by Alfred B. Thordarson

--

--------------------------------------------------------------------------------

1. INTRODUCTION

===============

This RTPC-model is mainly based on a verilog description, written at

CMU initially by Edward Czeck but Herman Schmit and Rich Cloutier have

made some changes which may or may not be included.

I have included a clock to clock the execution machine and a memory

latency time. This is the only timing in the model, there is no timing

on the execution of the instructions. This is just a behavioral-model

and it should not be used to time a sequence of instructions - but

merly to validate if the code works.

The model is written as one process and most of the instructions have

their own procedure which will be run when the instruction is carried

out. This is a very simple and easily understood model.

The model presents all the instruction set with the extension of the

"wait" instruction for run-time debugging and a nop-instruction (explained

below).

This model can not be used in synthesis at all and a model at the RT-

level should be extracted from this one before using any synthesis-tool.

This documentation is divided into several sub-sections. These

sections are:

1. Introduction

2. RTPC CPU

2.1 System and Ports

2.1.1 Control Unit

2.1.2 Memory Unit

2.1.3 RTPC-Model Unit

2.2 Execution Machine

2.3 Instruction Set

2.3.1 Instruction Formats

2.3.2 Table of Instructions

3. Run-Time Debugging / The Wait Instruction

4. Assembler / Running a Program

5. Testing

5.1 Testing Strategy

5.2 Tested Instructions

5.3 Testing Conclusions

2. RTPC CPU

===========

The RTPC-architecture is a 8-bit processor which has an 8 bit data-bus

and a 24 bit address-bus. Allocation of an array to hold the 24-bit

address-space takes an enormous time so the address space is kept at

20-bit - which helps alot. The address-size as well as the timing, mentioned

in the introduction, are generic parameters so they can easily be changed

in the system.vhd file.

2.1 System and Ports

~~~~~~~~~~~~~~~~~~~~

The system consists of three main-blocks. The Control Unit, the Memory

Unit and the RTPC-Model Unit. Any of these units can be extracted from this

configuration and used in another one. The execution of the program is done

by the RTPC-Model Unit which works in a Master-Slave mode with the Control

Unit. An explanation of each of the units follows.

2.1.1 Control Unit

~~~~~~~~~~~~~~~~~~

The control unit reads the file "program" and writes it to memory. The

Control then triggers the RTPC-Model to start execution and the RTPC-Model

informs the Control when the execution is complete.

GENERIC		TYPE		DESCRIPTION

--------------- --------------- ------------------------------------------

CLOCK_CYCLE	time		Time for one clock cycle.

MEM_LATENCY	time		Latency of a memory read or write.

ADDRESS_SIZE.	Decimal		Size of address-bus. Default 24 bits.

PORT	TYPE	BITS/ACTIVE	DESCRIPTION

------- ------- --------------- ------------------------------------------

CLOCK	InOut	1		Clock generation for the instr. execution

in the RTPC-Model Unit.

RESET	Out	1/HIGH		Reset-signal to the RTPC-Model Unit.

Initially HIGH, when the program is loaded

the RESET is set LOW for the RTPC-Model

Unit to start execution.

RWb	Out	1		Read(1)/Write(0) from/to memory

CSb	Out	1/LOW		Chip select for the memory

ADDRESS	Out	ADDRESS_SIZE	Address to memory

DATA	InOut	8		Data to/from memory

WAITING	In	1/HIGH		This signal is asserted by the RTPC-Model

to indicate that it has finished the

execution of the program.

2.1.2 Memory Unit

~~~~~~~~~~~~~~~~~

The memory unit used with the RTPC-model is a byte oriented memory with a

RWb and a CSb signal as most commercial memories.

GENERIC		TYPE		DESCRIPTION

--------------- --------------- ------------------------------------------

ADDRESS_SIZE.	Decimal		Size of address-bus. Default 32 bits.

PORT	TYPE	BITS/ACTIVE	DESCRIPTION

------- ------- --------------- ------------------------------------------

RWb	In	1		Read(1)/Write(0) from/to memory

CSb	In	1/LOW		Chip select

ADDRESS	In	ADDRESS_SIZE	Address of memory

DATA	InOut	8		Data to/from memory

2.1.3 RTPC-Model Unit

~~~~~~~~~~~~~~~~~~~~~

This is the execution machine which executes the RTPC-code starting at

address hex 40. The execution machine and the instruction set will covered

by the next two sections.

GENERIC		TYPE		DESCRIPTION

--------------- --------------- ------------------------------------------

MEM_LATENCY	time		Latency of a memory read or write.

ADDRESS_SIZE.	Decimal		Size of address-bus. Default 24 bits.

PORT	TYPE	BITS/ACTIVE	DESCRIPTION

------- ------- --------------- ------------------------------------------

CLOCK	InOut	1		External clock for the instr. execution

RESET	Out	1/HIGH		When the RESET-signal goes low the

execution starts at address hex 40 with

all registers and condition codes reset

to 0.

RWb	In	1		Read(1)/Write(0) from/to memory

CSb	In	1/LOW		Memory Chip select

ADDRESS	In	ADDRESS_SIZE	Address of memory

DATA	InOut	8		Data to/from memory

WAITING	Out	1/HIGH		This signal is asserted to indicate that

the execution has finished (wait-instr.

F0 FF - see section 3.2)

2.2 Execution Machine

~~~~~~~~~~~~~~~~~~~~~

The execution machine refers to the VHDL loop the simulator executes for

each of the RTPC-instruction. I will explain this by drawing a diagram which

will clarify how the VHDL loop works.

+------------------+

| INITIATE         |

| addr=0x40        |<----------------------------+

| wait for reset=0 |                             |

+------------------+                             |

|                                        |

V                                        |

+----------------------------------------+             |

| READ INSTRUCTION                       |             |

+--->| Instruction: 2 bytes at address addr   |             |

|    | Immidiate  : 2 bytes at address addr+2 |             |

|    +----------------------------------------+             |

|                    |                                      |

|                    V                                      |

|          +---------------------+      +-----------------+ |

|          | EXECUTE INSTRUCTION | <--> | INSTR.PROCEDURE | |

|          +---------------------+      +-----------------+ |

|                    |                                      |

|                    V                                      |

|                   /\                                      |

|                  /  \                                     |

|                 /    \  Yes                               |

|                /Offset\_________________                  |

|                \  =0? /                 |                 |

|                 \    /                  V                 |

|                  \  /        +------------------+         |

|                   \/         | RETURN CONTROL   |         |

|                    | No      | waiting=1        |         |

|                    |         | wait for reset=1 |         |

|                    V         +------------------+         |

|                    /\                  |                  |

|         Branch    /  \   Branch        +------------------+

|         Delayed  /    \  Now

|        _________/Branch\______________

|       |         \ Cond /              |

|       V          \    /               V

|  +-------------+  \  /          +------------------+

|  | BranchCond= |   \/           | addr=branch_addr |

|  | BranNow     |   | Branch     +------------------+

|  +-------------+   | None             |

|             |      |                  |

|             V      V                  |

|          +------------------+         |

|          | INCREMENT PC     |         |

|          | addr=addr+offset |         |

|          +------------------+         |

|                    |                  |

|<-------------------+------------------+

2.3 Instruction Set

~~~~~~~~~~~~~~~~~~~

This section both talks about the instruction formats and the

instructions them selfs. There is a table which has all the necessery

information to start making programs to be run on this model.

2.3.1 Instruction Formats

~~~~~~~~~~~~~~~~~~~~~~~~~

The RTPC-architectures instructions can have seven distinct formats:

0    5    8        15

_____________________

| OP | N  | J1      |        J1 Format

0    4    8    12  15

_____________________

| OP | R1 | R2 | R3 |        X Format

0    4    8    12  15

_____________________

| OP | I  | R2 | R3 |        D-Short Format

0         8    12  15

_____________________

| OP      | R2 | R3 |        R Format

0         8    12                      31

_________________________________________

| OP      | R2 | BI                     |        BI Format

0         8                            31

_________________________________________

| OP      | BA                          |        BA Format

0         8    12   16                 31

_________________________________________

| OP      | R2 | R3 | I                 |        D Format

To read VHDL-code with many instruction formats is very tedious

so I decided to use one general format throughout the VHDL-code.

I call this the General-VHDL Format:

0    4    8    12   16                 31

_________________________________________

|OP1 |OP2 | A  | B  | Immi              |        General-VHDL Format

2.3.2 Table of Instructions

~~~~~~~~~~~~~~~~~~~~~~~~~~~

In the table below A and B stand for registers and # means a number, so

the Rasm-format "LCS A,#(B)" means that the instruction LCS has three

parameters, two registers like R1, R5, R12 or R15 and a number. These

parameters should be written as the format suggests.

When reading the description gpA and gpB mean the General Purpose register

number A and B, besides this the table and the descriptions should be

pretty clear.

The RTPC-processor it self can execute all the following instructions,

which are grouped into ten classes. The VHDL model can also execute

all of them except for the marked ones. The reason the marked instructions

are not implemented is mentioned below the table.

1. MEMORY

Rasm-mnemonic	Op Size	Instruction			Description

---------------	-- ----	-----------------------------	---------------------------------------------------

LCS	A,#(B)	4   2	Load Character Short		gpA = Byte at mem addr (B=0)?0:gpB + #

LC	A,#(B)	CE  4	Load Character			gpA = Byte at mem addr (B=0)?0:gpB + #

LHAS	A,#(B)	5   2	Load Half Algebraic Short	gpA = Word at mem addr (B=0)?0:gpB + 2*#

LHA	A,#(B)	CA  4	Load Half Algebraic		gpA = Word at mem addr (B=0)?0:gpB + #

LHS	A,B	EB  2	Load Half Short			gpA = Word at mem addr gpB

LH	A,#(B)	DA  4	Load Half			gpA = Word at mem add (B=0)?0:gpB + #

LS	A,#(B)	7   2	Load Short			gpA = Long word at mem addr (B=0)?0:gpB + 4*#

L	A,#(B)	CD  4	Load				gpA = Long word at mem addr (B=0)?0:gpB + #

LM	A,#(B)	C9  4	Load Multiple			gp(A..15) = Long words beg at mem addr (B=0)?0:gpB + #

TSH	A,#(B)	CF  4	Test and Set Half		gpA = Word at mem addr (B=0)?0:gpB + # (mark word read)

STCS	#(B),A	1   2	Store Character Short		Write Byte from gpA to mem addr (B=0)?0:gpB + #

STC	#(B),A	DE  4	Store Character			Write Byte from gpA to mem addr (B=0)?0:gpB + #

STHS	#(B),A	2   2	Store Half Short		Write Word from gpA to mem addr (B=0)?0:gpB + 2*#

STH	#(B),A	DC  4	Store Half			Write Word from gpA to mem addr (B=0)?0:gpB + #

STS	#(B),A	3   2	Store Short			Write Long Word from gpA to mem addr (B=0)?0:gpB + 4*#

ST	#(B),A	DD  4	Store				Write Long Word from gpA to mem addr (B=0)?0:gpB + #

STM	#(B),A	D9  4	Store Multiple			Write Long Words from gp(A..15) to mem addr (B=0)?0:gpB + #

2. ADDRESS COMPUTATION

Rasm-mnemonic	Op Size	Instruction			Description

---------------	-- ----	-----------------------------	---------------------------------------------------

CAL	A,B,#	C8  4	Compute Address Lower Half	gpA = (B=0)?0:gpB + #

CAL16	A,B,#	C2  4	Compute Address Lower Half 16B	gpA = (B=0)?0:gpB + # (only lower 16 bits)

CAU	A,B,#	D8  4	Compute Address Upper Half	gpA = (B=0)?0:gpB + #

CAS	C,A,B	6   2	Compute Address Short		gpC = (B=0)?0:gpB + gpA

CA16	A,B	F3  2	Compute Address 16-Bit		gpA = gpA + gpB (only lower 16 bits)

INC	A,#	91  2	Increment			gpA = gpA + #

DEC	A,#	93  2	Decrement			gpA = gpA - #

LIS	A,#	A4  2	Load Immediate Short		gpA = #

3. BRANCHING

Rasm-mnemonic	Op Size	Instruction			Description

---------------	-- ----	-----------------------------	---------------------------------------------------

BALA	#	8A  4	Branch and Link Absolute	Branch to the even address # - immidiatly

BALAX	#	8B  4	Branch and Link Absolute X	Branch to the even address # - one delay slot

BALI	A,#	8C  4	Branch and Link I		Branch relativly to the even address # - immidiatly, gpA=curr.addr

BALIX	A,#	8D  4	Branch and Link I X		Branch to the even relativ address # - one delay slot, gpA=curr.addr

BALR	A,B	EC  2	Branch and Link			Branch to even address gpB - immidiatly, gpA=curr.addr

BALRX	A,B	ED  2	Branch and Link X		Branch to even address gpB - one delay slot, gpA=curr.addr

JB	#,#	0C1 2	Jump on Condition Bit		Jump to relativ # on # condition - immidiatly

BB	#,#	8E  4	Branch on Condition Bit I	Branch to relativ # on true # condition - immidiatly

BBX	#,#	8F  4	Branch on Condition Bit I X	Branch to relativ # on true # condition - one delay slot

BBR	#,A	EE  2	Branch on Condition Bit		Branch to even address gpA on true # condition - immidiatly

BBRX	#,A	EF  2	Branch on Condition Bit X	Branch to even address gpA on true # condition - one delay slot

JNB	#,#	0C0 2	Jump on Not Condition Bit	Jump to relativ # on # condition - immidiatly

BNB	#,#	88  4	Branch on Not Condition Bit I	Branch to relativ # on false # condition - immidiatly

BNBX	#,#	89  4	Branch on Not Condition Bit I X	Branch to relativ # on false # condition - one delay slot

BNBR	#,A	E8  2	Branch on Not Condition Bit	Branch to even address gpA on false # condition - immidiatly

BNBRX	#,A	E9  2	Branch on Not Condition Bit X	Branch to even address gpA on false # condition - one delay slot

I : Immediate, X : With Execute, OpCode C1 : Bits "1CCC" and C0 : Bits "0CCC" where CCC is number of condition

4. TRAPS

Rasm-mnemonic	Op Size	Instruction			Description

---------------	-- ----	-----------------------------	---------------------------------------------------

TI	#,A,#	CC  4	Trap on Condition Immediate	if gpA<=># then trap (condition depends on #)

TGTE	A,B	BD  2	Trap if Reg GreaterThan or Eq	if gpA>=gpB then trap

TLT	A,B	BE  2	Trap if Reg Less Than		if gpA<gpB then trap

5. MOVES AND INSERTS

Rasm-mnemonic	Op Size	Instruction			Description

---------------	-- ----	-----------------------------	---------------------------------------------------

MC03	A,B	F9  2	Move Character Zero From Three	gpA(byte 0) = gpB(byte 3)

MC13	A,B	FA  2	Move Character One From Three	gpA(byte 1) = gpB(byte 3)

MC23	A,B	FB  2	Move Character Two From Three	gpA(byte 2) = gpB(byte 3)

MC33	A,B	FC  2	Move Character Three From Three	gpA(byte 3) = gpB(byte 3)

MC30	A,B	FD  2	Move Character Three From Zero	gpA(byte 3) = gpB(byte 0)

MC31	A,B	FE  2	Move Character Three From One	gpA(byte 3) = gpB(byte 1)

MC32	A,B	FF  2	Move Character Three From Two	gpA(byte 3) = gpB(byte 2)

MFTB	A,B	BC  2	Move From Test Bit		Set a bit in gpA acording to gpB equal to test_bit

MFTBIL	A,#	9D  2	Move From Test Bit I Lower Half	Set a bit in gpA acording to # equal to test_bit

MFTBIU	A,#	9C  2	Move From Test Bit I Upper Half	Set a bit in gpA acording to #+16 equal to test_bit

MTTB	A,B	BF  2	Move to Test Bit		Set test_bit equal to a bit in gpA acording to gpB

MTTBIL	A,#	9F  2	Move to Test Bit I Lower Half	Set test_bit equal to a bit in gpA acording to #

MTTBIU	A,#	9E  2	Move to Test Bit I Upper Half	Set test_bit equal to a bit in gpA acording to #+16

I : Immediate

6. ARITHMETIC OPERATIONS

Rasm-mnemonic	Op Size	Instruction			Description

---------------	-- ----	-----------------------------	---------------------------------------------------

A	A,B	E1  2	Add				gpA = gpA + gpB

AE	A,B	F1  2	Add Extend			gpA = gpA + gpB + carry

AEI	A,B,#	D1  4	Add Extend Immediate		gpA = gpB + # + carry

AI	A,B,#	C1  4	Add Immediate			gpA = gpB + #

AIS	A,#	90  2	Add Immediate Short		gpA = gpA + #

ABS	A,B	E0  2	Absolute			gpA = abs(gpB)

ONEC	A,B	F4  2	Ones Complement			gpA = One's complement of gpB

TWOC	A,B	E4  2	Twos Complement			gpA = Two's complement of gpB

C	A,B	B4  2	Compare				Compare gpA and gpB

CIS	A,#	94  2	Compare Immediate Short		Compare gpA and #

CI	A,#	D4  4	Compare Immediate		Compare gpA and #

CL	A,B	B3  2	Compare Logical			Compare gpA and gpB (cond.code unsigned)

CLI	A,#	D3  4	Compare Logical Immediate	Compare gpA and # (cond.code unsigned)

EXTS	A,B	B1  2	Extend Sign			gpA = extended low word of gpB

S	A,B	E2  2	Subtract			gpA = gpA - gpB

SF	A,B	B2  2	Subtract From			gpA = gpB - gpA

SE	A,B	F2  2	Subtract Extended		gpA = gpB - gpA - carry

SFI	A,#,B	D2  4	Subtract From Immediate		gpA = # - gpB

SIS	A,#	92  2	Subtract Immediate Short	gpA = gpA - #

D	A,B	B6  2	Divide Step			Partial Division

M	A,B	E6  2	Multiply Step			Partial multiplication

7. LOGICAL OPERATIONS

Rasm-mnemonic	Op Size	Instruction			Description

---------------	-- ----	-----------------------------	---------------------------------------------------

CLRBL	A,#	99  2	Clear Bit Lower Half		Clear bit # in lower(gpA)

CLRBU	A,#	98  2	Clear Bit Upper Half		Clear bit # in upper(gpA)

SETBL	A,#	9B  2	Set Bit Lower Half		Set bit # in lower(gpA)

SETBU	A,#	9A  2	Set Bit Upper Half		Set bit # in upper(gpA)

N	A,B	E5  2	And				gpA = gpA and gpB

NILZ	A,B,#	C5  4	And Immediate Lower Half EX0	gpA = gpB and {0,#}

NILO	A,B,#	C6  4	And Immediate Lower Half EX1	gpA = gpB and {1,#}

NIUZ	A,B,#	D5  4	And Immediate Upper Half EX0	gpA = gpB and {#,0}

NIUO	A,B,#	D6  4	And Immediate Upper Half EX1	gpA = gpB and {#,1}

O	A,B	E3  2	Or				gpA = gpA or gpB

OIL	A,B,#	C4  4	Or Immediate Lower Half		gpA = gpB or {0,#}

OIU	A,B,#	C3  4	Or Immediate Upper Half		gpA = gpB or {#,0}

X	A,B	E7  2	Xor				gpA = gpA xor gpB

XIL	A,B,#	C7  4	Xor Immediate Lower Half	gpA = gpB xor {0,#}

XIU	A,B,#	D7  4	Xor Immediate Upper Half	gpA = gpB xor {#,0}

CLZ	A,B	F5  2	Count Leading Zeros		gpA = Count of leading zeros in gpB

EX0 : Extend Zeros, EX1 : Extend Ones

8. SHIFTS

Rasm-mnemonic	Op Size	Instruction			Description

---------------	-- ----	-----------------------------	---------------------------------------------------

SAR	A,B	B0  2	Shift Algebraic Right		gpA = gpA >> lowest 6 bits of gpB (extend sign)

SARI	A,#	A0  2	Shift Algebraic Right I		gpA = gpA >> # (extend sign)

SARI16	A,#	A1  2	Shift Algebraic Right I +16	gpA = gpA >> 16+# (extend sign)

SR	A,B	B8  2	Shift Right			gpA = gpA >> lowest 6 bits of gpB

SRI	A,#	A8  2	Shift Right I			gpA = gpA >> #

SRI16	A,#	A9  2	Shift Right I +16		gpA = gpA >> 16+#

SRP	A,B	B9  2	Shift Right Paired		gp(TWIN) = gpA >> lowest 6 bits of gpB

SRPI	A,#	AC  2	Shift Right Paired I		gp(TWIN) = gpA >> #

SRPI16	A,#	AD  2	Shift Right Paired I +16	gp(TWIN) = gpA >> 16+#

SL	A,B	BA  2	Shift Left			gpA = gpA << lowest 6 bits of gpB

SLI	A,#	AA  2	Shift Left I			gpA = gpA << #

SLI16	A,#	AB  2	Shift Left I +16		gpA = gpA << 16+#

SLP	A,B	BB  2	Shift Left Paired		gp(TWIN) = gpA << lowest 6 bits of gpB

SLPI	A,#	AE  2	Shift Left Paired I		gp(TWIN) = gpA << #

SLPI16	A,#	AF  2	Shift Left Paired I +16		gp(TWIN) = gpA << 16+#

I : Immediate, +16 : Plus Sixteen

9. SYSTEM CONTROL

Rasm-mnemonic	Op Size	Instruction			Description

---------------	-- ----	-----------------------------	---------------------------------------------------

MTS	A,B	B5  2	Move to SCR			scrA = gpB

MFS	A,B	96  2	Move From SCR			gpA = scrB

CLRSB	A,#	95  2	Clear SCR Bit			clear bit # in scrA

SETSB	A,#	97  2	Set SCR Bit			set bit # in scrA

WAIT	#	F0  2	Wait				Used for debugging and to return control to control unit

LPS		D0  4	Load Program Status				*** Not Implemented

SVC		C0  4	Supervisor Call					*** Not Implemented

NOP		0   2	No operation			Does absolutly nothing

10. INPUT/OUTPUT

Rasm-mnemonic	Op Size	Instruction			Description

---------------	-- ----	-----------------------------	---------------------------------------------------

IOR		CB  4	Input/Output Read				*** Not Implemented

IOW		DB  4	Input/Output Write				*** Not Implemented

Instructions MTS, MFS, CLRSB and SETSB are instructions to work on the

System Control Registers (SCR). There are 15 scr's in the RTPC-architecture

but most of them are reserved or not important to us. We have implemented

two important scr's, the Multiplier Quotient (s10) and the Condition Status

(s15). So these registers can be used with these instructions but other

registers are not implemented.

The implemented part of the SCR is

0               16            31

________________________________

SCR10	|                              |        Multiplier Quotient

SCR15	|XXXXXXXXXXXXXXX|XXXXXXX     X |        Condition Status

||||| ||

||||| |31:TestBit

||||| 30:Overflow

||||28:CarryZero

|||27:GreaterThan

||26:EqualTo

|25:LessThan

24:PermanentZero

Instructions that reference a condition-code, like conditional branches,

specify the condition-code with a 4 bit number. To find out what condition-

code is being tested 16 should be added to that number. In the JB and JNB

instructions the condition-code number is only 3 bit. To find out what

condition-code is being tested 24 should be added to that number.

The Input/Output instructions are not implemented yet, but will probably

be implemented in a future release. There is no actual reason for this

except that these instructions are used to transfer data between the General-

Purpose registers and registers in the MMU. As we are not using an MMU then

why spend time on implementing these instructions.

LPS and SVC are instructions to control program status, interrupts and

other stuff that are used in a general purpose operating system. We are not

conserned with these functions in this model.

The documentation of the RTPC-architecture that I have does not indicate

what will happen if instructions JB and JNB are to jump the distance of 0

(on to themselfs). This would make and infinite loop so instead of this I have

the model do nothing (the NOP instruction). I used the NOP-instruction

in the Branch/Jump-test when I couldn't find an instruction to fit into the

delayed branch instruction slots

3. RUN-TIME DEBUGGING / THE WAIT INSTRUCTION

============================================

There is an instruction called "wait" which can be used for runtime debugging

or to return control to the control unit. I have utilized this instruction

so that you can call it with various parameters and get one of the following

written out to the file called "dump.registers":

- A line to distinguise between individual debug-calls

- Condition codes

- Contents of a part of or all the registers

The instruction has the following format:

F0 XX

|  ||

|  |Register to start from

|  |

|  Bit 3   - Should a line be printed

|  Bit 2   - Should the condition codes be printed

|  Bit 1,0 - Number of registers to be printed - 1;

|

The instruction code

If the "Register to start from" plus "Number of registers to be printed - 1"

is equal to 16 then all registers are printed but if the sum exceeds that

then non is. To stop the running and return control to the control unit you

use the instruction format F0 FF.

Some examples are:

* F0 00 = No line, No cond.codes, 0+1=1 registers starting at reg 0

* F0 01 = No line, No cond.codes, 0+1=1 registers starting at reg 1

* F0 02 = No line, No cond.codes, 0+1=1 registers starting at reg 2

* F0 80 = Pr.Line, No cond.codes, 0+1=1 registers starting at reg 0

* F0 86 = Pr.Line, No cond.codes, 0+1=1 registers starting at reg 6

* F0 B6 = Pr.Line, No cond.codes, 3+1=4 registers starting at reg 6 (6,7,8 and 9)

* F0 F6 = Pr.Line, Pr.cond.codes, 3+1=4 registers starting at reg 6 (6,7,8 and 9)

* F0 FC = Pr.Line, Pr.cond.codes, 3+1=4 registers starting at reg C (C,D,E and F)

* F0 FD = Pr.Line, Pr.cond.codes, 3+1=4 registers starting at reg D (D+3 = 16 => All registers printed)

* F0 FE = Pr.Line, Pr.cond.codes, 3+1=4 registers starting at reg E (E+3 = 17 => No  registers printed)

* F0 FF = Print nothing, just return control to control-unit

4. ASSEMBLER / RUNNING A PROGRAM

================================

In this section I would like to explain how to write a program to be run

and debugged with the RTPC-model.

What you want to do is load the memory with one of your programs, then

have the model run on your code and last but not least you want to be able

to verify outcome. With the tools, that come with this model, you are able

to do all that and as the previous section explained, even use run-time

debugging.

First of all you make a file with the extension ".asm", this file is to

contain your program and all the debugging and control information needed

for the model to execute the code and view the results just the way you

want it.

Each line of the file may have some information for the control unit or

it may be a comment. The first letters of each line indicate if the line

contains control-information, then line begins with one of the following:

- "@"		A line starting with this symbol is a line

which changes the address at which code

is compared or written to memory.

- "wri"		Oposite to "com", that is the following

code is to be written to memory at the

indicated addresses. This is the default

when the model starts running

- "run"		Instructs the RTPC-model to start running.

The model runs until it hits an wait

instruction with the parameters FF.

- "com"		The following data should be compared

to memory at the indicated addresses.

If there are some errors then they are

written to the file dump.errors

- "mem"		Dump a part of the memory to the file

dump.memory, i.e. mem 40 67 dumps then

contents of addresses hex 40 to hex 67.

- "qui"		Simulation is to stop completly.

If a line does not start with one of the above keywords then the line must

contain code or data to be written or compared to memory. If the control unit

can't extract code nor data from the line it is discarded as being comment.

The code lines have the format indicated in the instruction table above.

Lines that contain data should start with a Double Quote (") or a Equal-

Sign (=).

I would like to indicate that all numbers in the ".asm" file are considered

decimal numbers unless they start with the "0x"-prefix, in which case they are

treated as hexadecimal.

Last but not least, when an end of file is encountered the simulation is

stopped completly just as if there had been a quit statement in the file.

Lets look at an example. To run a program which subtracts two numbers your

".asm"-file would could something like:

1	--------------------------------------------------------------------------------

2	--

3	-- RTPC CPU Benchmark :

4	--	Demo program used in the documentation

5	--	Subtracts two numbers twice and stores the results

6	--

7	-- Derived from

8	--	ROMP description written in verilog by Edward Czeck et al.

9	--	Most likely based on the IBM RT-PC Hardware Technical

10	--	Reference (c) 1985 (for RT PC model 10, 20, and 25)

11	--

12	-- Authors:

13	--	Alfred B. Thordarson (abth@ece.uci.edu)

14	--	and

15	--	Nikil Dutt, professor of CS and ECE

16	--	University Of California, Irvine, CA 92717

17	--

18	-- Changes:

19	--	Dec 1, 1993: File created by Alfred B. Thordarson

20	--

21	--------------------------------------------------------------------------------

22

23	write to memory

24

25	@ 0x40	-- Code

26

27	cau	r2,r0,1

28	l	r2,0(r2)

29	cau	r3,r0,1

30	l	r3,4(r3)

31	s	r2,r3

32	cau	r3,r0,1

33	st	8(r3),r2

34	wait	0xfd

35

36	cau	r2,r0,1

37	l	r2,0x10(r2)

38	cau	r3,r0,1

39	l	r3,0x14(r3)

40	s	r2,r3

41	cau	r3,r0,1

42	st	0x18(r3),r2

43	wait	0xfd

44

45	wait	0xff

46

47	@ 0x10000

48

49	-- Operands A and B and space for result FF FF FF FF

50	=00 00 00 01

51	=00 00 00 02

52	=00 00 00 00

53

54	@ 0x10010

55

56	-- Operands A and B and space for result 80 00 23 FF

57	=0x7f 0xff 0xf0 00

58	=0xff 0xff 0xcc 01

59	=00 00 00 00

60

61	run

62

63	compare to memory

64

65	@ 0x10008

66	=0xff,0xff,0xff,0xff

67	@ 0x10018

68	=0x80,0,0x23,255

69

70	mem 0x10000 0x1001B

Don't let the line-numbers confuse you, they are merly there to help

me reference parts of the code in the following explanation.

* The first lines, 1 through 21, obviously would be recognized as comments

because they don't start with any of the 6 control-keywords, a mnemonic,

a Equal-sign nor a Double Quote, the same thing goes for all empty lines

in the file.

* The first keyword is in line 23. The "wri" means what the line spells out

that the following code should be written to memory.

* The keyword "@" in line 25 then sets the address to hex 40 so the control-

unit will start writing to that address.

* Lines 27 to 45 then hold the code. This code will be written to addresses

hex 40 and as far as it needs.

* The next line in the sequence is 47, which includes a new address.

* The data in lines 50 to 52 will be written in addresses hex 10000 to

hex 1001B.

* The keyword "run" in line 61 then tells the control-unit to transfer

control to the RTPC-model that always starts running at address hex 40. The

model then runs the code we just put to memory. When the RTPC-model hits

the command wait 0xff (return control) it returns control to the control-

unit which continues down the file.

* Line 63 is the next line to be considered when the code has been run. It

contains the keyword "com" which means that the following code should be

compared to memory.

* Line 65 then holds the address to be compared and line 66 holds the data.

This data is then compared to memory, starting at address hx 10008.

* Line 67 holds another address and line 68 the data to be compared to

addresses hex 10018 to 1001B. If there are any errors in the comparisons the

errors are written to the file "dump.errors".

* Line 70 is the last line to be considered. It contains the keyword "mem"

which dumps a part of the memory to file. The part to be dumped is between

addresses hex 10000 to hex 1001B. This is dumped to the file "dump.memory".

* Finally when the control unit hits the end of the file it asserts a FAILURE

and the simulation finishes. If you use the command in the README-file, to

run the simulation, this assertation is written to the file "run.out" - just

like any other assertions that may appear during your simulation.

From this overview you should get the general idea of how to write a file that

includes a program to be run on the RTPC-model. The file "demo.asm" contains the

file above.

When you have written your program you should follow the guidelines in the

README-file to simulate it. The assembler gives you a "program"-file that

looks something like:

1	WRI

2	@ 40

3	D8 20 00 01

4	CD 22 00 00

5	D8 30 00 01

6	CD 33 00 04

7	E2 23

8	D8 30 00 01

9	DD 23 00 08

10	F0 FD

11	D8 20 00 01

12	CD 22 00 10

13	D8 30 00 01

14	CD 33 00 14

15	E2 23

16	D8 30 00 01

17	DD 23 00 18

18	F0 FD

19	F0 FF

20	@ 10000

21	00 00 00 01

22	00 00 00 02

23	00 00 00 00

24	@ 10010

25	7F FF F0 00

26	FF FF CC 01

27	00 00 00 00

28	RUN

29	COM

30	@ 10008

31	FF FF FF FF

32	@ 10018

33	80 00 23 FF

34	MEM 10000 1001B

As you can see this is a compact file, where your code has been translated

to hexadeciaml numbers. If the simulation doesn't work as you expected you

can run the assembler with the command "rasm -d file.asm" and it will make

the "program"-file with all your comments and explanations of what code is

generated for each line. This would look something like:

1	--------------------------------------------------------------------------------

2	--

3	-- RTPC CPU Benchmark :

4	--	Demo program used in the documentation

5	--	Subtracts two numbers twice and stores the results

6	--

7	-- Derived from

8	--	ROMP description written in verilog by Edward Czeck et al.

9	--	Most likely based on the IBM RT-PC Hardware Technical

10	--	Reference (c) 1985 (for RT PC model 10, 20, and 25)

11	--

12	-- Authors:

13	--	Alfred B. Thordarson (abth@ece.uci.edu)

14	--	and

15	--	Nikil Dutt, professor of CS and ECE

16	--	University Of California, Irvine, CA 92717

17	--

18	-- Changes:

19	--	Dec 1, 1993: File created by Alfred B. Thordarson

20	--

21	--------------------------------------------------------------------------------

22

23	WRI

24

25	@ 40

26

27	D8 20 00 01	// cau r2,r0,1

28	CD 22 00 00	// l r2,0(r2)

29	D8 30 00 01	// cau r3,r0,1

30	CD 33 00 04	// l r3,4(r3)

31	E2 23      	// s r2,r3

32	D8 30 00 01	// cau r3,r0,1

33	DD 23 00 08	// st 8(r3),r2

34	F0 FD      	// wait 0xfd

35

36	D8 20 00 01	// cau r2,r0,1

37	CD 22 00 10	// l r2,0x10(r2)

38	D8 30 00 01	// cau r3,r0,1

39	CD 33 00 14	// l r3,0x14(r3)

40	E2 23      	// s r2,r3

41	D8 30 00 01	// cau r3,r0,1

42	DD 23 00 18	// st 0x18(r3),r2

43	F0 FD      	// wait 0xfd

44

45	F0 FF      	// wait 0xff

46

47	@ 10000

48

49	-- Operands A and B and space for result FF FF FF FF

50	00 00 00 01

51	00 00 00 02

52	00 00 00 00

53

54	@ 10010

55

56	-- Operands A and B and space for result 80 00 23 FF

57	7F FF F0 00

58	FF FF CC 01

59	00 00 00 00

60

61	RUN

62

63	COM

64

65	@ 10008

66	FF FF FF FF

67	@ 10018

68	80 00 23 FF

69

70	MEM 10000 1001B

Now, from this file you can examine more clearly what the assembler

did. From this file you should be able to see if you made a mistake

in your Rasm-format or if the assembler is not working correctly.

5. TESTING STRATEGY

===================

Ofcourse there is no way to test this model completly because then

we would have to test all combinations of instructions and all possible

combinations of parameters. I don't even know if the piece of hardware

we are modelling here would pass a test like that if it could be performed.

5.1 Testing strategy

~~~~~~~~~~~~~~~~~~~~

The best we can do to test if the model is working, as well as we expect,

we can have it run all the instructions. We can try each instruction with

one, two or three diffirent parameters to see if all the instructions run

properly. That is exacly what the "valid_*.asm" files do. Each

file tests a specific part of the instruction set. The instructions that

are tested with each of the files can be seen in the table below.

5.2 Tested instructions

~~~~~~~~~~~~~~~~~~~~~~~

The names of the files all start with "valid_" to indicate that the

files are used to validate the model. They all end with ".asm" to

indicate that these are assembly code files that have to be assemled

before they will be run on the model. To assemble each of the files you

do "rasm valid_XXX" which will make the correct "program"-file and then

you can start the simulator as shown in the README-file. The instructions

tested in each file are:

FileName	Instruction-Groups		Comments

---------------	-------------------------------	--------------------

valid_mem.asm	1. Memory			Loads and stores

2. Address Computation

valid_jmp.asm	3. Branching

valid_mov.asm	5. Moves and inserts	Uses MFS

valid_log.asm	7. Logical Operations

8. Shifts

valid_ari.asm	6. Arithmetic Operations	Except for multiply and divide

valid_msd.asm	6. Arithmetic Operations	Multiply and Divide

9. System Control

valid_trp.asm	4. Traps

The files are assembled and simulated in this order to make sure all

instructions used in each file are eather from the instruction being

tested or from instruction that have already been tested and proved to

work. But even if this is done I could run into problems with instructions

that should have been tested earlier. For an example the first file should

validate all memory-accesses but if I do a load short and then a store

short to another address, the result could be correct even though the

load didn't load into the correct part of the designated register. But

later on, in the move instructions I could run into problems with this

if a short load instruction doesn't load into the correct byte, I would

then detect the error during the execution of the "valid_mov.asm"-file and

correct it at that time. That's the best I can do......

If some errors arise during simulation the errors will be written

to one of the files "dump.asserts" og "dump.errors".

5.3 Testing Conclusions

~~~~~~~~~~~~~~~~~~~~~~~

Lets look at some characterisitcs that the Rasm assembler gives us:

FileName	Instr/Bytes	Data	Comp	Control	RealTime/User/Sys

---------------	---------------	----	----	-------	-----------------

valid_mem.asm	159/444		152	240	8	46.2/42.0/ 2.3

valid_jmp.asm	111/306		0	92	6	23.8/20.3/ 2.0

valid_mov.asm	83/232		0	108	6	23.7/18.8/ 3.1

valid_log.asm	144/444		0	144	6	34.6/30.7/ 2.1

valid_ari.asm	176/566		0	148	6	40.3/36.0/ 2.6

valid_msd.asm	71/162		0	16	6	16.6/13.0/ 1.8

valid_trp.asm	8/28		0	0	4	 6.8/ 3.2/ 2.3

---------------	---------------	----	----	-------	-----------------

Total		748/2174	152	748	42	 192/ 164/16.2

Don't think that 748 instructions gives an average of 7 instances of

each of the 118 instructions. This is incorrect because the INC, ST,

CAL and CAU are used very often to store and make values. I would guess

that this testing tested each instruction on the average of 1.3 instances

and that can and should be improved.

The time was measured with the UNIX-command "time" on the SUN-

computer Clyde on the ICS-net at the University of California, Irvine.

<div align="center"><br /><script type="text/javascript"><!--

google_ad_client = "pub-7293844627074885";

//468x60, Created at 07. 11. 25

google_ad_slot = "8619794253";

google_ad_width = 468;

google_ad_height = 60;

//--></script>

<script type="text/javascript" src="http://pagead2.googlesyndication.com/pagead/show_ads.js">

</script><br />&nbsp;</div>