AUTHOR: H. Fatih Ugurdag, Staff Fellow (hugurdag@bonnie.cs.gmr.com)

ORGANIZATION: GM R&D Center, Warren, MI

GOAL OF SYNTHESIS:

Simulation results of the synthesized circuit SHOULD match with

simulation results of the behavior on a clock-cycle-by-clock-cycle

basis.

DESCRIPTION of Behavior:

This is a "period counter." It counts the length of a complete

cycle of signal "IN" in terms of the number of "Clock" cycles it

takes. The circuit is asleep as long as "reset" is low. When reset

is released, it waits for a positive edge on "COMMAND" (1st while

loop). Note that the circuit assumes that the initial value of

COMMAND is 0. That is if COMMAND is 1 the first time it is sampled,

that is regarded as a positive edge. Then, the circuit waits for a

positive edge on IN (2nd while loop), because we want to measure a

complete cycle of IN. That is, we have to start with a fresh new

cycle of IN. Then, we count the positive half cycle of IN (3rd

while loop).Then, we keep counting in the negative half cycle of IN

(4th while loop). Then, we update the output "PERIOD." The circuit

later starts waiting for a new positive edge on COMMAND.

The interesting feature of this behavioral description is the

locations of Clock edges. There is no clock edge before we enter

while loops (except the 1st). And there is no clock edge before we

exit a while loop.

Most behavioral synthesis tools won't even accept this

description.

<div align="center"><br /><script type="text/javascript"><!--

google_ad_client = "pub-7293844627074885";

//468x60, Created at 07. 11. 25

google_ad_slot = "8619794253";

google_ad_width = 468;

google_ad_height = 60;

//--></script>

<script type="text/javascript" src="http://pagead2.googlesyndication.com/pagead/show_ads.js">

</script><br />&nbsp;</div>