Department of Electrical and Computer Engineering

COEN 212                                                                                  Dec.15, 2016

Answer all Questions.                                                 Exam Duration 3 hour

Examiners : A. J. Al-Khalili and S. Das

No books / papers or electronic devices are allowed.

===============================================================

- Question  1

For a) and b) below use Boolean Algebra.

a)Given F1 and F2 below, determine optimal (F1 . F2) and (F1 + F2)	 (2 Marks)

F1= AB + C’

F2= A’C’ + B’C’

b) Minimize the following Boolean Function:			( 2Marks)

F(A,B,C,D)= ABC’ + BC’D’ + AC’D + ABC + BCD’ + ACD’

c) Given   F(A,B,C) = AB +A C’ + BC				(3 Marks)

i) Implement f in NOR-NOR format

ii) Implement f in AND-OR-INVERT format

Obtain optimum implementation.

Question 2

a) Design a digital combinational circuit that implements			(8 Marks)

F = (N-2)2   where   N is an  integer   0  N   6

Show your design steps clearly starting with the Truth Table.

Draw the final circuit.

b) What size of a ROM is required for the implementation of F?

(2 Marks)

Question 3

- Using a 2 to 1 MUX implement AND, OR and XOR gates.        (1 Marks)

- Implement the Full Adder given below using minimum 2 to 1 MUXes only:

S (A,B,C)  = A B  C

Cout (A,B,C) = A . B + C . (A  B )                           ( 4 Marks)

- Using a 4 to 1 MUX implement F

F (A,B,C,D) =  A’.B + A .B.D + A.B’. D’                ( 3 Marks)

Question 4

Design a Decoder that receives a BCD number and displays a P on a 7-segment display as shown in Fig. 1 below. If a non-BCD number appears at the input an F should be displayed.  Start with a truth table and derive final output. Give final circuit diagram.     (8 marks)

a

b               c

d

e               f

g

a

b

c

d

e

f

g

w

x

Y

z

BCD

Fig. 1

Decoder                                                   Display

Question 5

Design an UP/DOWN BCD counter, starting with a state diagram. Use T-Flip Flop for your implementation.			(7 Marks)

## Question 6

- Analyze the circuit given in Fig. 2 below fully (give excitation equations, transition table, state table and state diagram):

- Initially Do has been set to “1” (for one clock period). Draw the timing diagram for y0, y1, y2 for 5 consecutive clock cycles. Assume each Flip flop has a delay of 10 ns. (10 marks)

y2

y1

y0

CLK

Doo0

D1

D2

Fig. 2. Circuit to be analyzed in Q.6.